Application Cases of Three-in-One Equipment in Different Fields

I. Pharmaceutical Industry (Aseptic Active Pharmaceutical Ingredients)

| Case Subject | Core Requirements | Equipment Configuration | Implementation Effect | Compliance Certification |
| --- | --- | --- | --- | --- |
| Union Pharmaceutical(a leading domestic manufacturer of aseptic APIs) | 1. Long production cycle of antibiotic APIs (8 hours/batch with traditional process)2. High risk of cross-contamination during material transfer, making it difficult to pass GMP certification3. Large solvent loss (300L/batch) | Aseptic three-in-one equipment:1. Material: 316L stainless steel (mirror finish, Ra≤0.4)2. Agitation: Full-cavity lifting paddle (double mechanical seal)3. Drying: Vacuum + jacket heating (≤60℃)4. Auxiliary: CIP/SIP online cleaning and sterilization, OEB4 level closed system | 1. Production cycle shortened to 4.5 hours, batch output increased by 20%2. Cross-contamination risk reduced by 95%, OEE reached 95%3. Solvent loss reduced by 40%, saving over 2 million yuan annually4. Successfully passed GMP/FDA on-site certification | GMP, FDA 21 CFR Part 210/211 |

II. Fine Chemical Industry (Pesticide/Dye Intermediates)

| Case Subject | Core Requirements | Equipment Configuration | Implementation Effect | Compliance Certification |
| --- | --- | --- | --- | --- |
| A large fine chemical enterprise (Yangtze River Delta region) | 1. Strongly acidic pesticide intermediates (pH=1) corrode equipment2. Low washing efficiency and high impurity residue3. Large consumption of washing water, leading to heavy environmental pressure | Customized three-in-one equipment by Shanghai Lüjing Machinery:1. Material: Duplex stainless steel 2205 (high corrosion resistance)2. Filtration medium: Sintered titanium filter element (10μm)3. Agitation: Anchor type with scraper (explosion-proof variable frequency)4. Auxiliary: Multi-stage displacement washing + solvent recovery module | 1. Continuous operation for 72 hours without material corrosion2. Solid-liquid separation efficiency improved by 35%, impurity residue reduced to below 0.01%3. Washing water consumption reduced by 28%, wastewater discharge decreased by 30% | ATEX explosion-proof, environmental emission ≤30mg/m³ |

III. New Energy Materials Industry (Lithium Battery Cathode Materials)

| Case Subject | Core Requirements | Equipment Configuration | Implementation Effect | Compliance Standard |
| --- | --- | --- | --- | --- |
| A leading lithium battery material enterprise (Guangdong Province) | 1. Excessive impurities (Fe, Cu) in ternary materials (NCM)2. Susceptible to oxidation, oxygen content difficult to control (＞0.3% with traditional process)3. High drying energy consumption, water content ＞0.2% | Guoji General MY Series three-in-one equipment:1. Material: Titanium alloy (resistant to electrolyte corrosion)2. Filtration medium: Basket-type sintered mesh (filtration efficiency ≥95%)3. Drying: Vacuum + nitrogen protection (oxygen content ≤0.2%)4. Auxiliary: Online oxygen content analysis + automatic pulping and washing | 1. Impurity removal rate ≥99%, product purity reaches 99.9%2. Oxygen content stably controlled between 0.15%-0.25%3. Water content reduced to below 0.1%, energy consumption decreased by 25%4. Batch cycle shortened from 20 hours to 10 hours | GB/T 30038 (Lithium Battery Material Standard) |

IV. Food Industry (Functional Ingredients)

| Case Subject | Core Requirements | Equipment Configuration | Implementation Effect | Compliance Standard |
| --- | --- | --- | --- | --- |
| Probiotic powder manufacturer(Shandong Province) | 1. Thermosensitive materials (probiotics) are prone to inactivation at high temperatures2. Traditional drying equipment has many sanitary dead corners, resulting in excessive total bacterial count3. Target water content ≤5% | Customized hygienic compact three-in-one equipment:1. Material: Food-grade 316L stainless steel (inner wall Ra≤0.8)2. Filtration medium: Food-grade polypropylene filter membrane (5μm)3. Drying: Vacuum low temperature (≤45℃)4. Auxiliary: 100% coverage CIP cleaning + online foreign matter detection | 1. Probiotic survival rate increased by 30%, active ingredient retention rate ≥95%2. Total bacterial count ≤10CFU/g, meeting food hygiene standards3. Final water content stably maintained at 3%-4%, product shelf life extended by 6 months | GB 16798 (Food Contact Material Standard) |

V. Semiconductor/Hydrogen Energy Industry (High-Purity Materials)

| Case Subject | Core Requirements | Equipment Configuration | Implementation Effect | Compliance Standard |
| --- | --- | --- | --- | --- |
| Huadian Engineering Green Hydrogen Project(Damao Banner, Inner Mongolia) | 1. Hydrogen purity for fuel cells needs to reach 99.999%2. Removal of trace moisture (dew point ≤-70℃) and particles (＜0.1μm) required3. Adapt to fluctuating working conditions of wind/photovoltaic hydrogen production | Ultra-high purity three-in-one purification equipment:1. Material: High-purity 316L + PTFE coating (zero precipitation)2. Filtration medium: Nanoceramic membrane (0.01μm)3. Drying: Vacuum + ultra-low dew point drying (dew point ≤-70℃)4. Auxiliary: Online particle counter + high-purity gas purification system | 1. Hydrogen purity reaches 99.999%, residual oxygen content below 1ppmv2. Particle retention rate ≥99.9%, fully meeting fuel cell requirements3. Zero hydrogen loss, fully automatic operation without manual intervention | ISO 14687-2 (Hydrogen Energy Standard), SEMI F47 |

VI. Environmental Protection Industry (Hazardous Waste Treatment)

| Case Subject | Core Requirements | Equipment Configuration | Implementation Effect | Compliance Standard |
| --- | --- | --- | --- | --- |
| Chemical hazardous waste treatment enterprise(Jiangsu Province) | 1. Spray waste contains corrosive solvents, prone to leakage during treatment2. Large volume of solid waste, urgent need for volume reduction3. Low solvent recovery efficiency, heavy environmental pressure | Explosion-proof three-in-one equipment:1. Material: Hastelloy (high corrosion resistance)2. Agitation: Ribbon-type high-power stirring (explosion-proof servo motor)3. Drying: Vacuum + waste heat recovery4. Auxiliary: Solvent recovery module (recovery rate ≥90%) | 1. Fully closed operation, zero solvent leakage, meeting environmental emission requirements2. Solid waste volume reduced by 60%, achieving volume reduction target3. Solvent recovery rate reaches 92%, annual recovered solvent value exceeds 1 million yuan4. Continuous operation service life of equipment reaches over 8000 hours | Environmental emission ≤30mg/m³, ATEX explosion-proof |