Application Fields and Selection Guide of Three-in-One Equipment

With core advantages of closed integration, low pollution, high efficiency and controllable energy consumption, three-in-one equipment has been deeply applied in multiple industrial fields featuring high added value, high purity, and stringent requirements for environment and safety. Its core application fields and specific scenarios are as follows:

I. Pharmaceutical Industry (Core Application Field)

- 1.Sterile API Production: Such as β-lactam antibiotics, carbapenem drugs, and hormonal APIs. The equipment must comply with GMP/FDA/EMA certifications and meet OEB4 closed system standards, achieving zero leakage of organic solvents and zero dust emission. It completes the entire process of feeding-filtration-washing-drying-discharging in a single closed container, avoiding cross-contamination during material transfer.

- 2.Traditional Chinese Medicine (TCM) Extract Processing: For materials like TCM extract powder and botanical extracts, it realizes low-temperature vacuum drying to retain the activity of active ingredients, while reducing solvent residues to below 50 ppm.

- 3.Pharmaceutical Excipient Preparation: Such as purification of microcrystalline cellulose, lactose and other excipients. Multi-stage washing is adopted to improve purity and meet pharmaceutical standards.

II. Fine Chemical Industry

- 1.Pesticide Intermediates: Such as herbicide and insecticide intermediates. It is suitable for strongly acidic, strongly alkaline and corrosive materials, adopting titanium/double-phase steel materials. It supports multiple displacement washing to reduce impurity content and improve product yield.

- 2.Dyestuffs and Pigments: Such as phthalocyanine dyes and organic pigments. Precise control of drying temperature and stirring speed ensures uniform particle size of powder, prevents caking, and meets application requirements of high-end coatings and inks.

- 3.Specialty Chemicals: Such as catalysts, flame retardants and electronic-grade chemicals. It requires high-cleanliness operation to prevent the introduction of metal ion impurities, and the purity of dried products can reach over 99.9%.

III. New Energy Materials Industry (High-growth Application Field)

- 1. Lithium Battery Cathode Materials: Such as ternary materials (NCM) and lithium iron phosphate. High-cleanliness design reduces impurities like iron and copper; drying temperature is precisely controlled within ±1℃ to ensure electrochemical performance of materials. Nitrogen-protected drying can stably control the oxygen content of products within 0.15%-0.25%.

- 2.Solid-state Battery Electrolytes: For oxide and sulfide electrolyte powders, it realizes washing and drying in a closed low-oxygen environment, avoiding reaction of electrolytes with air and ensuring product stability.

- 3.Photovoltaic Materials: Such as treatment of by-products from polysilicon purification, and washing and drying of photovoltaic silver paste precursors, meeting the stringent requirements of the photovoltaic industry for high purity and low impurities.

IV. Polymer Materials Industry

- 1.Engineering Plastics: Such as high-performance resin powders including PEEK, PA12 and PEN. Closed operation prevents oxidation and moisture absorption; the stirring system ensures powder fluidity; solvent residues are controlled below 10 ppm, suitable for 3D printing, high-end coatings and other fields.

- 2.Functional Polymers: Such as ion exchange resins and polymer flocculants. Unreacted monomers are removed through multiple washing; the pore structure of resins is maintained after drying to improve adsorption performance.

V. Food and Environmental Protection Industry

- 3.Food Deep Processing: Such as purification of vegetable protein, probiotic powder and functional dietary fiber. Low-temperature vacuum drying retains nutrients; fully closed operation complies with food-grade hygiene standards.

- 4.Environmental Hazardous Waste Treatment: Such as volume reduction treatment of chemical hazardous waste and spraying waste. Organic solvents are recovered through filtration and washing; the volume of solid waste is reduced after drying to realize resource utilization. It can also be used for deep dehydration and drying of sludge in industrial wastewater treatment.

VI. High-precision Fields Such as Hydrogen Energy and Semiconductors (Emerging Expansion Fields)

- 1.Hydrogen Energy Industry: Purification and drying of high-purity hydrogen for fuel cells. It is required to remove trace moisture and particles (particle size < 0.1 μm), ensuring hydrogen purity reaches over 99.999% to meet fuel cell operation requirements.

- 2.Semiconductor Materials: Such as purification of photoresist raw materials and electronic-grade cleaning agents. High-cleanliness (Class 100) and corrosion-resistant design avoids particle contamination, supporting semiconductor chip manufacturing processes.

Selection Comparison Table of Three-in-One Equipment for Different Application Fields

| Selection Dimension | Pharmaceutical Industry Sterile APIs | Fine Chemical Industry Pesticides & Dyestuffs | New Energy Materials Industry Lithium Battery Cathodes | Food Industry Functional Ingredients | Semiconductor & Hydrogen Energy High-purity Materials |
| --- | --- | --- | --- | --- | --- |
| Core Materials | 316L stainless steel (mirror finish Ra≤0.4), sterile-grade sealing | Titanium/double-phase steel/Hastelloy, glass-lined | 316L/titanium alloy, resistant to electrolyte corrosion | Food-grade 304/316L, residue-free design | High-purity 316L + PTFE coating, zero-precipitation materials |
| Filtration Medium | Multi-layer sintered mesh (1–5μm), metal bellows seal | Sintered titanium/ceramic filter elements, acid and alkali resistant | Basket-type sintered mesh, filtration efficiency ≥95% | Food-grade polypropylene filter membrane, sterilizable | Nano-ceramic membrane (0.01–0.1μm), Class 100 cleanliness |
| Stirring System | Full-cavity paddle, double-end dry mechanical seal, liftable | Anchor/ribbon type, wear-resistant, variable frequency speed regulation | Scraper-equipped stirring, nitrogen protection, anti-oxidation | Frame-type paddle, food-grade lubrication, leak-free | Magnetic coupling stirring, zero leakage, low particle generation |
| Drying Method | Vacuum + jacket heating, low-temperature drying ≤60℃ | Vacuum/hot air, adjustable temperature 80–120℃ | Vacuum + nitrogen purging, oxygen content ≤0.2% | Vacuum low-temperature ≤50℃, retains activity | Vacuum + ultra-clean hot air, dew point ≤-70℃ |
| Containment Level | OEB4, leakage rate ≤0.0013bar/min | OEB3, zero solvent leakage | OEB3, positive pressure 0.02–0.05MPa | Sanitary-grade seal, 100% CIP coverage | OEB5, fully closed + online leak detection |
| Key Control Parameters | Temperature ±0.5℃, vacuum degree ≤1kPa, batch fluctuation ≤±2% | Temperature ±1℃, precise control of washing liquid dosage | Moisture content ≤0.1%, impurity removal rate ≥99% | Moisture ≤5%, total bacterial count ≤10CFU/g | Particle size ≤0.1μm, purity ≥99.999% |
| Special Configurations | CIP/SIP, sterile sampling, PLC compliant with GAMP5 | Multi-stage displacement washing, explosion-proof design, solvent recovery | Online oxygen content analysis, automatic slurry washing | Online foreign matter detection, material traceability | Online particle counter, high-purity gas purification system |
| Compliance Standards | GMP/FDA/EMA, OEB4, sterility validation | ATEX explosion-proof, environmental emission ≤30mg/m³ | GB/T 30038, lithium battery material industry standard | GB 16798, food contact material standard | SEMI F47, ISO 14687-2 (hydrogen energy) |
| Typical Models | Sterile cylinder-cone type (500L–5m³) | Corrosion-resistant closed type (1–10m³) | High-cleanliness customized type (2–8m³) | Sanitary compact type (0.5–3m³) | Ultra-high-purity customized type (0.1–2m³) |

Key Points for Equipment Selection

- 1.Material Matching: For corrosive materials, prioritize titanium/glass-lined materials; for sterile scenarios, 316L mirror finish + sterile sealing is mandatory; for high-purity scenarios, zero-precipitation materials are required.

- 2. Filtration Precision: 1–5μm for APIs, 5–10μm for lithium battery materials, and nanoscale filtration for semiconductors.

- 3.Drying Efficiency: Select vacuum low-temperature drying for heat-sensitive materials; add nitrogen protection for oxidation-prone materials; adopt ultra-low dew point drying for high-purity materials.

- 4.Compliance Adaptation: Pharmaceutical applications require CIP/SIP and GMP validation; chemical applications require explosion-proof and solvent recovery functions; food applications require sanitary design and traceability.

Procurement and Selection Reference

| Process Scale & Batch Volume | Material Characteristics Corrosiveness/Oxidizability | Target Final Moisture Content | Recommended Model | Core Configuration | Suitable Industry | Compliance Standards |
| --- | --- | --- | --- | --- | --- | --- |
| Lab-scale Pilot Test 50L–200L | Weakly corrosive (e.g., TCM extracts, food ingredients) | ≤5% | JG-50/200 Sanitary Compact Type | 1. Material: Food-grade 304 stainless steel (inner wall Ra≤0.8)2. Filtration medium: Polypropylene filter membrane (5μm)3. Stirring: Frame-type paddle, variable frequency speed regulation (0–60r/min)4. Drying: Vacuum low-temperature (≤50℃), dead-angle-free heating5. Auxiliary: Manual discharging, simple cleaning system | Food (probiotic powder, dietary fiber), TCM extracts | GB 16798 (food contact materials) |
| Pilot-scale Expansion 300L–800L | Moderately corrosive (e.g., pesticide intermediates, dyestuffs) | ≤1% | SY-300/800 Corrosion-resistant Closed Type | 1. Material: Duplex steel 2205 (acid and alkali resistant)2. Filtration medium: Sintered titanium filter element (10μm)3. Stirring: Anchor-type with scraper, explosion-proof variable frequency motor4. Drying: Vacuum + jacket heating (80–120℃ adjustable)5. Auxiliary: Solvent recovery module, online washing liquid metering | Fine chemicals (pesticides, dyestuff intermediates) | ATEX explosion-proof, environmental emission ≤30mg/m³ |
| Large-scale Production 1m³–5m³ | Strongly corrosive (e.g., lithium battery cathode materials, pharmaceutical intermediates) | ≤0.1% | ZT-1/5 Sterile High-cleanliness Type | 1. Material: 316L stainless steel (mirror finish Ra≤0.4) + sterile-grade metal bellows seal2. Filtration medium: Multi-layer sintered mesh (1–5μm), basket-type structure3. Stirring: Full-cavity liftable paddle, double-end dry mechanical seal4. Drying: Vacuum + nitrogen protection (oxygen content ≤0.2%), temperature precisely controlled at ±0.5℃5. Auxiliary: CIP/SIP online cleaning and sterilization, PLC control system (compliant with GAMP5) | Pharmaceuticals (sterile APIs), new energy (ternary materials/lithium iron phosphate) | GMP/FDA/EMA, OEB4 containment |
| Ultra-large Capacity 8m³–15m³ | Strongly corrosive + oxidation-prone (e.g., specialty chemicals, lithium battery cathode materials) | ≤0.05% | GD-8/15 Industrial Customized Type | 1. Material: Titanium/Hastelloy (resistant to strong acid and alkali)2. Filtration medium: Ceramic filter element (5μm), backwash regeneration function3. Stirring: Ribbon-type high-efficiency stirring, explosion-proof servo motor4. Drying: Vacuum + waste heat recovery, nitrogen circulation purging5. Auxiliary: Automatic discharging, online moisture detection, intelligent process traceability system | Fine chemicals (large-scale intermediates), new energy (10,000-ton lithium battery production lines) | GB/T 30038 (lithium battery standard), ATEX explosion-proof |
| Ultra-high Purity Field 100L–500L | Ultra-high purity + slightly corrosive (e.g., photoresist raw materials, fuel cell hydrogen purification) | ≤0.01% | CC-100/500 Ultra-high Purity Customized Type | 1. Material: High-purity 316L + PTFE coating (zero precipitation)2. Filtration medium: Nano-ceramic membrane (0.01–0.1μm), Class 100 cleanliness3. Stirring: Magnetic coupling drive (zero leakage), low particle generation4. Drying: Vacuum + ultra-low dew point drying (dew point ≤-40℃)5. Auxiliary: Online particle counter, high-purity gas purification system | Semiconductors (photoresist), hydrogen energy (fuel cell hydrogen) | SEMI F47, ISO 14687-2 |

Supplementary Selection Recommendations

- 1.For heat-sensitive materials (e.g., biological agents, probiotics), an additional low-temperature cooling system should be  installed to control the drying temperature within 30–40℃.

- 2.For fully automated production requirements, upgrade configurations with robot discharging + MES system integration to  realize unmanned operation and data traceability.

- 3.For flammable and explosive materials, confirm the equipment's explosion-proof rating (Ex d IIB T4 is recommended), and  install inert gas protection interlock devices.